# Project ARGUS - Final ML Pipeline and Execution Plan

An AI-Powered Smart Glasses System for the Visually Impaired

Two-Speed Vision-Language Architecture with Gemma 4 E2B and YOLO-World (revision incorporating Phase-0 feasibility findings).

Deployment platform: NVIDIA Jetson Orin Nano Super (8 GB). Perception: 2x Arducam AR0234 stereo pair + 1x Arducam IMX477P wide camera.

Note: This .docx is a text transcription of the design document for in-repo searchability. The authoritative version with diagrams and tables is docs/ARGUS_Final_Pipeline.pdf.

# 1. Executive Summary

This document finalises the ML pipeline for Project ARGUS following a Phase-0 feasibility investigation into on-device deployment of the two candidate foundation models. Google Gemma 4 E2B is well supported on the Jetson Orin Nano Super and runs with multimodal (image) input via a native llama.cpp build. NVIDIA LocateAnything-3B cannot presently be deployed on this device: it ships BF16-only, exposes no INT4/GGUF/TensorRT export path, relies on a custom Parallel Box Decoding head whose kernel is restricted to Hopper/Blackwell GPUs, and is documented as targeting datacentre GPUs and the larger Jetson Thor. The grounding role is therefore reassigned from LocateAnything-3B to YOLO-World, an open-vocabulary detector that runs natively under TensorRT on the Orin Nano at interactive speed. The Two-Speed architecture is preserved in full.

## Final Locked Decisions

Architecture: Two-Speed Vision-Language design (non-ML safety reflex + event-driven VLM agent). Unchanged.

Reasoning agent: Google Gemma 4 E2B, multimodal, via native llama.cpp. Confirmed deployable.

Grounding tool: YOLO-World (open-vocabulary), via TensorRT. Replaces LocateAnything-3B.

Safety reflex loop: non-ML, rule-based, continuously running. Unchanged.

Privacy gate: mandatory face/text blur before any frame reaches the agent. Unchanged.

# 2. Phase-0 Feasibility Findings

## 2.1 Gemma 4 E2B - Confirmed deployable

Vendor support: NVIDIA and Google published Day-0 support for Gemma 4 E2B on the Jetson Orin Nano, including a reference agent combining STT, the Gemma 4 vision model with camera input, and TTS on a single 8 GB Orin Nano Super.

Inference path: native llama.cpp using a quantised GGUF checkpoint plus the official vision projector file, exposing an OpenAI-compatible endpoint that accepts image input.

Measured performance: ~11 tokens/s decode at INT4 in high-power mode, resident footprint ~3-4.5 GB.

## 2.2 LocateAnything-3B - Not presently deployable on Orin Nano

Weights are BF16 only; no INT4 build or published quantisation recipe.

Model card: TensorRT, TensorRT-LLM and Triton not yet supported (PyTorch/transformers only).

Parallel Box Decoding uses Python-level hybrid control flow - no GGUF/ONNX/TensorRT export route.

MagiAttention kernel is Hopper/Blackwell only; falls back to slow PyTorch attention on Ampere.

Supported hardware: A100/Hopper/Lovelace/Blackwell and Jetson Thor - Orin Nano not included.

Decision: Adopt YOLO-World as the grounding tool. Open-vocabulary detection, runs natively under TensorRT on the Orin Nano at ~160 ms per query, leaving ample memory for the resident agent.

# 3. Revised Component Stack

| Role | Model / Method | Loop | Deployment |
| --- | --- | --- | --- |
| Reasoning agent | Gemma 4 E2B (multimodal) | Slow | llama.cpp (GGUF, INT4); optional LoRA |
| Grounding tool | YOLO-World | Slow | TensorRT (INT8/FP16); light fine-tune |
| Stereo depth | RAFT-Stereo / SGBM | Fast | TensorRT |
| Obstacle & hazard | Geometric rules | Fast | Native code |
| SLAM | OpenVINS / ORB-SLAM3 | Fast | Native code |
| Privacy gate | SCRFD + CRAFT | Slow | TensorRT |
| Wake word | openWakeWord | - | CPU |
| Speech-to-text | faster-whisper (tiny) | - | CPU (INT8) |
| Text-to-speech | Piper | - | CPU |

# 4. The Two-Speed Architecture

A continuously running, non-ML fast loop guarantees physical safety; an event-driven slow loop provides intelligent interaction on demand. Only the grounding tool invoked by the agent differs.

## Worked interaction: find my keys

1. The fast loop is already running, tracking obstacles and pose continuously and independently.

2. The user speaks; the wake word triggers and faster-whisper transcribes find my keys.

3. The wide-camera frame passes through the privacy gate (faces blurred) and is presented to Gemma 4 E2B with the query.

4. Gemma 4 issues a function call to YOLO-World with the open-vocabulary class keys.

5. YOLO-World returns a bounding box in ~160 ms; the box centre is sampled against the depth map for a 3D position.

6. Gemma 4 converts the position into guidance and Piper speaks the result.

# 5. YOLO-World as the Grounding Tool

Open vocabulary: detects arbitrary, user-named objects via text prompts at inference time, no retraining.

Edge-deployable: standard ONNX and TensorRT export paths; runs on the Orin Nano Ampere GPU without exotic kernels.

Fast: ~160 ms per query under TensorRT vs several seconds for a 3B VLM grounder.

Light: a few hundred MB resident, coexisting with the Gemma 4 agent within 8 GB.

Alternative: NanoOWL (TensorRT OWL-ViT) retained as a fallback.

# 6. Compute Resources and Operating Rules

## 6.2 GPU selection rule

Writing / debugging -> Local GTX 1060 or Colab CPU (no cost).

First test of a new notebook / routine inference / export -> T4.

T4 out-of-memory -> L4 (cheaper than A100).

Heavier fine-tune -> A100, only after a 1-epoch T4 profiling pass.

## 6.3 Non-negotiable efficiency rules

Never train or read datasets directly from Google Drive; copy archives to local session storage first.

Always use mixed precision for any training/fine-tuning run (on Colab; FP32 locally on Pascal).

Profile one epoch on a low-cost accelerator before scaling; utilisation below 80% indicates a data-pipeline problem.

Checkpoint to persistent storage at regular intervals, never only at the end.

Release the accelerator immediately on completion of a run.

Apply early stopping; do not train to completion out of habit when validation loss has plateaued.

## 6.4 Platform constraints (Phase 0)

llama.cpp Python bindings will not build on Colab Python 3.12; LM inference is performed on the Jetson.

Audio libraries such as pyaudio will not build on Colab; audio handling is performed on the Jetson.

TensorRT engines are device-specific and must be built on the Jetson; Colab produces ONNX only.

All ONNX exports use dynamo=False, as the PyTorch 2.x dynamo exporter fails on certain vision operators.

# 8. On-Device Deployment and Memory Budget

With YOLO-World replacing the 3B grounder, all components are resident simultaneously, no model swapping. Approx budget: Gemma 4 E2B (INT4 GGUF) ~2.0-2.5 GB; YOLO-World (FP16/INT8) ~0.3-0.5 GB; stereo depth (INT8) ~0.2 GB; SLAM ~0.5 GB; privacy gate ~0.2 GB; speech (CPU) ~0.1 GB; OS/CUDA/buffers ~1.5 GB. Total ~4.8-5.5 GB - within 8 GB with headroom.

# 9. Phased Execution Roadmap

Phase 0: Feasibility verification - Complete.

Phase 1: YOLO-World validation, fine-tune, ONNX export (T4/L4) - 1-2 weeks.

Phase 2: Privacy gate integration (T4) - 1 week.

Phase 3: Gemma 4 agent + tool contract (Jetson) - 2 weeks.

Phase 4: Stereo depth + safety reflex + SLAM (Local/Jetson) - 2 weeks.

Phase 5: Speech pipeline integration (Jetson) - 1 week.

Phase 6: Full on-device integration (Jetson) - 2-3 weeks.

Phase 7: HCI study with participants - 4-6 weeks.

# 11. Summary of Changes

Grounding tool: LocateAnything-3B -> YOLO-World via TensorRT. Grounder deployability: key open risk -> resolved. Grounding latency: several seconds -> ~160 ms. Memory budget: ~6.5-7.0 GB worst case with swapping -> ~4.8-5.5 GB all resident, no swapping. Reasoning agent: Gemma 4 E2B (unchanged, now confirmed). Architecture and privacy obligations: unchanged.